YL-12000T Pneumatic Unlock

Installation & Operation

Manual

Special Notes.

▲ Damage to the equipment caused during shipment shall be claimed from the carrier.

▲ has been designed and manufactured. However, proper training and careful operations can also increase safety. The equipment shall not be operated or repaired without reading the instructions.

▲ identifies the power and current status on the motor nameplate and requires a professional and qualified electrician.

▲ To ensure life safety and avoid electric shock accidents, please ensure that all grounding locations are reliable.

▲ Company does not inform the local structural improvement of the product. There is no obligation for the previously sold product update.

▲ Please carefully read and fill in the Warranty Card on the final page of this manual and feed back to the dealer and the Company for filing. As one of the basis for providing after-sales service. Otherwise, it shall be deemed to automatically waive the right to enjoy the corresponding services and assume the responsibility.

▲ The shall not exceed the rated lifting weight of 27000 lbs (12.2T) load of the equipment.

▲ Read warning signs on certified devices.

Table of contents

1. Main Performance of the Lift.........................................................................................................pg 4

- Basic parameters for the Lift,........................................................................................................pg 4

- Overall dimensions of the Lift,....................................................................................................pg5

- Installation of lift...........................................................................................................................pg 5

5. Lift debugging................................................................................................................................pg 6

6. Maintenance and Inspection of lift................................................................................................pg 7

7. Parts decomposition diagram and schedule of Lift........................................................................pg 8

- Hydraulic system and electrical elements of the lift....................................................................pg52

- 9. Common troubleshooting method..................................................................................................pg57

- 10. Packing list...................................................................................................................................pg58

- 11. List of vulnerable parts:................................................................................................................pg59

- Main performance of the Lift

● Double parking space parking elevator, beautiful appearance, save site space.

● Motor drive oil pump drive hydraulic cylinder, wire rope drive, quiet working environment.

● Mechanical safety lock, can be arbitrarily locked within the design range of height for operation, safe and reliable.

● Four corners can be adjustable in the locking position.

- Basic parameters for the Lift

| Model: | A | B | C | D | E | F | G | H | I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12000T -6,550 m | 2365mm | 220mm | 1675mm | 3533mm | 3303mm | 4003mm | 2930mm | 1830mm | 610mm |
| 12,000T -7500 m | 2365mm | 220mm | 1675mm | 3533mm | 3303mm | 4003mm | 2930mm | 1830mm | 610mm |
| J |  | K | L | M | N | O | P | Load load | Power supply |
| 955mm | 1330mm | 4003mm | 3303mm | 7792mm | 6810mm | 6010mm | 6620mm | 27,000 lbs | 240/50HZ,4.0KW |
| 955mm | 1330mm | 4003mm | 3303mm | 8742mm | 7760mm | 6960mm | 7570mm | 27,000 lbs | 240V/50HZ,4.0KW |

Test load:                13.7t

Test rise time:             91s

Test drop time:             27s

Test air pressure:            0.6mpa

Test the Hydraulic Pressure:    26mpa

- Overall dimensions of the lift

Refer to (Figures I and Figure 2).

- Installation of the lift

Step 1:  Before selecting the site to install the new lift, note the following:
1.)The position of lift shall be based to the requirements of the design planning plan of the whole field.
2.) determines the installation location and lines it according to the foundation drawing size of Figure 1.

3.) determines that the ground is free of any defects and guarantees the foundation mix strength to 3000psi (2.1kg/m m)

4.) leave the box for missing parts and damage in transport. According to the packing list.

Step 2:  Install columns and beams (Z-1,Z-2) and install fuse plates and sliders.(Z-2,Z-2-1,Z-2-2).

Step 3:  Install crossbeam and runway, wire rope, baffle and shield. (Z-3,Z-3-1,Z-3-2,Z-3-3,Z-3-4).

Step 4:  Install the power unit, tubing, and pneumatic fittings (Z-4,Z-4-1).

Step 5:  Install the boarding board, rubber brake rubber pad.(Z-5).

Step6:  Connect the power supply and add hydraulic oil according to the requirements on the motor nameplate.

Step7:  Point up and observe whether the position of the wire rope is correct. Electromagnet unlock insurance down the runway to the lowest point, adjust the tension of the wire rope to make the four consistent. Connect the power supply and add hydraulic oil according to the requirements on the motor nameplate.

Step 8:  Fixed Z-4-2 s.

Step 9:  Adjust the position between the column components and the cross beam. Limit blocks are required to be close to the column. Adjust the perpendicular alignment of the column to the ground plane with iron inserts. Beat anchor bolt holes and fastening bolts.

Step 10:  Adjust the runway level. Lift a certain height to lock it out. Measure the level, adjust the nut to the required position and lock.

Step 11:  Post all labels and finishing.

- Operation of the lift

☞ The fills the power unit fuel tank with pressurized oil (hydraulic oil plate No.: N32 or N46).

☞ The points the rise start button to cause the runway frame to rise and remove the support frame. Pay special attention to observe the wire rope during the first point movement

Inside the wheel, take the correct position.

☞ Press the drop button to keep the elevator in the normal drop lock, and observe whether the lock block can effectively enter the square hole of the lifting bar.

☞ descent runway frame to the lowest position and adjust the nuts on the four ropes to the same tension.

☞ no-load test run up and down twice to check whether all parts work normally. Check the track locking reliability, repeat several times and check whether the hydraulic system works normally.

Special attention to the lift

▲ Vehicles parked on the runway have to brake and use tire wedges.

▲ When lifting and lowering vehicles, note that the surrounding people and objects should not stay close to the lift.

▲ Four columns shall be perpendicular to the ground plane after installation, otherwise there will be danger in use!

▲Before the descent operation, the lock block can leave the lifting bar square hole. Otherwise will cause damage to the operating mechanism.

- Instructions for the maintenance and inspection of the lift

Routine maintenance and inspection

- Check the working position of the safety lock during the operation

- Check the unlocking and padlock of the safety lock and post fuse

- Check the hydraulic connections, air pipe connections and hose for leakage

- Check the wire rope connection if —— is bent, broken and loose.

- Check wear during rope lifting.

- Check all card spring connections and be connected reliably.

- Check all connections of bolts, nuts and screws ——, tighten immediately

- Check the wiring and switches for damage.

- Check the foundation strength around the expansion bolts.

- The shshaft shall be lubricated at least once or regularly after the lift is service
  
Weekly maintenance and Inspection

- Check the strength and connection of expansion bolts and tighten immediately if found loose

- Check the foundation strength around the expansion bolts

- Check the hydraulic oil level height

- Check and tighten the loose bolts, nuts, and screws

- Check all range wheel and range shaft alignment

Monthly maintenance and Inspection

- Lubricate the cable wheel and the cable wheel shaft

- Check the wear of wire rope and replace immediately with serious wear

- Replace the hydraulic oil.

7.Parts decomposition diagram and schedule of lift

Figure 1

Figure II

1-2-3-4

5-6

7-8-11-12-13-14

9-10-15-64

16-17-18-19-20-21-22-23-24-25-26-27-28-29

30-31-32-33-34-35-36-37-38-39-40-41-42-43-44-45-46-47-48-49-50

51-102-103-105

52-53-56-58-59-60-61-70-71-72-73

55-57-62-65-66-68

68-69-76-77-78-83-84-85-86-87

74-79-80-81-82

75-88-89-90-91-92-93-94-95-96-97-98-104-106

99-100-101

ZPT-1

ZPT-1-1

ZPT-1-2

ZPT-1-3

ZPT-1-4

ZPT-2

ZPT-2-1

ZPT-2-2

ZPT-3

Z-1

Z-2

Z-2-1

Z-2-2

Z-2-3

Z-3

Z-3-1

Z-3-2

Z-3-3

Z-4

Z-4-1

Z-4-2

Z-5

Z-6

Parts schedule

| Master number | Sub-serial number | Product Name | Quantity / machine |
| --- | --- | --- | --- |
| 27000-Pneumatic unlock |  |  |  |
| Welding part |  |  |  |
| 1 |  | Main column assembly | 1 |
| 2 |  | Auxiliary column-A assembly | 1 |
| 3 |  | Auxiliary column-B assembly | 2 |
| 4 |  | Column fuse plate assembly | 4 |
| 5 |  | Main platform assembly | 1 |
| 6 |  | Auxiliary platform assembly | 1 |
| 7 |  | Main platform oil cylinder front rope wheel shaft assembly | 1 |
| 8 |  | Main platform oil cylinder rear rope wheel shaft assembly | 1 |
| 9 |  | cylinder rear beam assembly | 1 |
| 10 |  | Cylinder front cross-beam assembly | 1 |
| 11 |  | Cross am rope wheel shaft | 4 |
| 12 |  | Beam mechanical safety hook | 4 |
| 13 |  | Cross-bar pneumatic insurance hook A | 2 |
| 14 |  | Cross-bar pneumatic insurance hook B | 2 |
| 15 |  | Car board assembly | 2 |
| Standard parts |  |  |  |
| 16 |  | M5X15 round-head bolt | 8+8 |
| 17 |  | M6 × 15 Flat-head screw | 16 |
| 18 |  | M6 × 15 External hexagon bolt | 2 |
| 19 |  | M6X15 round-head bolt | 32 |
| 20 |  | M6 × 20 round-head bolt | 2+4+2 |
| 21 |  | M6 × 25 External hexagon bolt | 8 |
| 22 |  | M8 × 15 round-head bolt | 8 |
| 23 |  | M8 × 15 External hexagon bolt | 4 |
| 24 |  | M8 × 35 flat-head screw | 8 |
| 25 |  | M8X35 outer hexagon bolt | 4 |
| 26 |  | M12 × 25 Flat-head screw | 24 |
| 27 |  | M12 × 30 external hexagon bolt | 4+2 |
| 28 |  | M14 × 50 outer hexagon bolt | 8 |
| 29 |  | M14 × 60 outer hexagon bolt | 8 |
| 30 |  | Diameter: 5 flat pad | 8+8 |
| 31 |  | Diameter: 5 bullet pad | 8 |
| 32 |  | Diameter: 6 flat pad | 4+2+2+32 |
| 33 |  | Diameter: 6 bullet pad | 4+2+2+32 |
| 34 |  | Diameter: 8 flat pad | 8+8+4 |
| 35 |  | Diameter: 8 bullet pad | 8+8+4 |
| 36 |  | Diameter: 12 flat pad | 4+2 |
| 37 |  | Diameter: 12 bullet pad | 4+2 |
| 38 |  | Diameter: 14 flat pad | 16 |
| 39 |  | Diameter: 14 bullet pad | 16 |
| 40 |  | Diameter: 24 flat pad | 8 |
| 41 |  | Diameter: 30 flat pad | 8 |
| 42 |  | M5 nut | 8 |
| 43 |  | M6 nut | 4 |
| 44 |  | M6 self-locking nut | 8 |
| 45 |  | M8 nut | 4 |
| 46 |  | M8 self-locking nut | 8 |
| 47 |  | M10 butter tter | 2+4 |
| 48 |  | M24 nut | 8 |
| 49 |  | M30 nut | 8 |
| 50 |  | M30 self-locking nut | 1 |
| Column assembly part |  |  |  |
|  | 1 | Main column assembly | 1 |
|  | 2 | Auxiliary column-A assembly | 1 |
|  | 3 | Auxiliary column-B assembly | 2 |
| 51 |  | Power unit | 1 |
| 52 |  | 9 / 16 Tip power unit connector | 1 |
| 53 |  | ￠ 14 Copper sealing gasket | 1 |
|  | 25 | M8X35 outer hexagon bolt | 4 |
|  | 34 | Diameter: 8 flat pad | 8 |
|  | 35 | Diameter: 8 bullet pad | 4 |
|  | 45 | M8 nut | 4 |
| 54 |  | Central electric control box (power selection and distribution box part) | 1 |
|  | 20 | M6 × 20 round head bolt (power distribution box section) | 4 |
|  | 32 | Diameter 6 flat pad (distribution distribution box part) | 4 |
|  | 33 | Diameter 6 bullet pad (distribution distribution box part) | 4 |
|  | 43 | M6 nut (power distribution box section) | 4 |
| 55 |  | AL-3000-03 large oil and water separator | 1 |
| 56 |  | Right-angle air connector for ￠ 8 | 2 |
|  | 20 | M6 × 20 round-head bolt | 2 |
|  | 32 | Diameter: 6 flat pad | 2 |
|  | 33 | Diameter: 6 bullet pad | 2 |
| 57 |  | Oil and water separator protection bracket | 1 |
|  | 18 | M6 × 15 External hexagon bolt | 2 |
|  | 32 | Diameter: 6 flat pad | 2 |
|  | 33 | Diameter: 6 bullet pad | 2 |
| 58 |  | TSV86522-S pneumatic unlock panel valve | 1 |
| 59 |  | Pass-through air connector for ￠ 8 | 2 |
| 60 |  | Micro-silencer | 2 |
| 61 |  | Plug head | 1 |
| 62 |  | ￠ 8, trachea | 1 |
| 63 |  | Travel switch (optional and power distribution box section) | 2 |
|  | 16 | M5X15 round head bolt (distribution distribution box section) | 8 |
|  | 30 | Diameter 5 flat pad (distribution distribution box part) | 8 |
|  | 31 | Diameter 5 bullet pad (distribution distribution box part) | 8 |
|  | 4 | Column fuse plate assembly | 4 |
|  | 40 | Diameter: 24 flat pad | 8 |
|  | 48 | M24 nut | 8 |
|  | 27 | M12 × 30 external hexagon bolt | 4 |
|  | 36 | Diameter: 12 flat pad | 4 |
|  | 37 | Diameter: 12 bullet pad | 4 |
| Platform assembly part |  |  |  |
|  | 5 | Main platform assembly | 1 |
|  | 6 | Auxiliary platform assembly | 1 |
| 64 |  | Oil cylinder | 1 |
| 65 |  | Cylinl shaft | 1 |
| 66 |  | Diameter 60 card spring | 2 |
| 67 |  | Steel ire pull plate | 1 |
| 68 |  | Wire rope sliding slide | 2 |
|  | 20 | M6 × 20 round-head bolt | 2 |
| 69 |  | Steel wire rope locking plate |  |
|  | 50 | M30 self-locking nut | 1 |
| 70 |  | 90 ° ZG3/8 internal and outer connector | 1 |
| 71 |  | ZG3/8 tooth explosion-proof valve | 1 |
| 72 |  | 9 / 16 Tip tip | 1 |
| 73 |  | Diameter: 16 copper pad | 2 |
| 74 |  | Double head 9 / 16 tip oil pipe is always one bend | 1 |
|  | 62 | ￠ 8, trachea | 3 |
| 75 |  | Three-gas connector for ￠ 8 | 1 |
| 76 |  | Platform steel wire rope wheel | 6 |
| 77 |  | Nylon wear sleeve | 6 |
| 78 |  | Main platform rope wheel gasket | 4 |
|  | 7 | Main platform oil cylinder front rope wheel shaft assembly | 1 |
|  | 8 | Main platform oil cylinder rear rope wheel shaft assembly | 1 |
|  | 47 | M10 butter tter | 2 |
|  | 27 | M12 × 30 external hexagon bolt | 2 |
|  | 36 | Diameter: 12 flat pad | 2 |
|  | 37 | Diameter: 12 bullet pad | 2 |
| 79 |  | Type of cord core 17.5, L1 | 1 |
| 80 |  | Type of cord core 17.5, L2 | 1 |
| 81 |  | Type of cord core 17.5, L3 | 1 |
| 82 |  | Type of cord core 17.5, L4 | 1 |
|  | 49 | M30 nut | 8 |
|  | 41 | Diameter: 30 flat pad | 8 |
|  | 28 | M14 × 50 outer hexagon bolt | 8 |
|  | 29 | M14 × 60 outer hexagon bolt | 8 |
|  | 40 | Diameter: 14 flat pad | 16 |
|  | 41 | Diameter: 14 bullet pad | 16 |
| Cross am assembly section |  |  |  |
|  | 9 | cylinder rear beam assembly | 1 |
|  | 10 | Cylinder front cross-beam assembly | 1 |
| 83 |  | Cross am insurance pull spring bracket A | 2 |
| 84 |  | Cross am insurance pull spring bracket B | 2 |
|  | 22 | M8 × 15 round-head bolt | 8 |
|  | 34 | Diameter: 8 flat pad | 8 |
|  | 35 | Diameter: 8 bullet pad | 8 |
| 85 |  | Beam steel wire rope wheel | 4 |
| 86 |  | Oil-free bearing | 4 |
|  | 11 | Cross am rope wheel shaft | 4 |
| 87 |  | Beam rope wheel retaining ring | 8 |
|  | 23 | M8 × 15 External hexagon bolt | 4 |
|  | 34 | Diameter: 8 flat pad | 4 |
|  | 35 | Diameter: 8 bullet pad | 4 |
|  | 47 | M10 butter tter | 4 |
| 88 |  | Cross-beam wire rope limit pin | 4 |
| 89 |  | Diameter 12 card spring | 8 |
| 90 |  | Cross am safety shaft | 4 |
|  | 12 | Beam mechanical safety hook | 4 |
|  | 13 | Cross-bar pneumatic insurance hook A | 2 |
|  | 14 | Cross-bar pneumatic insurance hook B | 2 |
| 91 |  | Cross am mechanical insurance limit pin | 4 |
| 92 |  | Beam mechanical insurance limit wheel | 4 |
| 93 |  | Beam beam slider B-100X30X6 | 16 |
|  | 17 | M6 × 15 Flat-head screw | 16 |
| 94 |  | Pull the spring (￠ 1.6 × ￠ 12 × 72) | 8 |
| 95 |  | LMSAL16 × 45-Y safe cylinder with Y-type connector | 4 |
| 96 |  | Direct air connector for ￠ 8 (cylinder type) | 4 |
|  | 21 | M6 × 25 External hexagon bolt | 8 |
|  | 44 | M6 self-locking nut | 8 |
|  | 75 | Three-gas connector for ￠ 8 | 2 |
|  | 62 | ￠ 8, trachea | 4 |
| 97 |  | Beam beam slider A | 8 |
|  | 26 | M12 × 25 Flat-head screw | 24 |
| 98 |  | Beam Limit Slider- ￠ 50 ￠ 12 × 11 | 8 |
|  | 24 | M8 × 35 flat-head screw | 8 |
|  | 46 | M8 self-locking nut | 8 |
| 99 |  | Small cross of shield | 4 |
| 100 |  | Long shield of crossmember | 2 |
| 101 |  | Long shield of beam | 4 |
|  | 19 | M6X15 round-head bolt | 32 |
|  | 32 | Diameter: 6 flat pad | 32 |
|  | 33 | Diameter: 6 bullet pad | 32 |
| Complete assembly part |  |  |  |
|  | 15 | Car board assembly | 2 |
| 102 |  | Nylon slip | 2 |
|  | 16 | M5X15 round-head bolt | 8 |
|  | 30 | Diameter: 5 flat pad | 8 |
|  | 42 | M5 nut | 8 |
| 103 |  | Car board pin | 2 |
| 104 |  | R-type pin | 4 |
| 105 |  | Fender plate | 2 |
| 106 |  | Rubber brake rubber pad | 2 |

8. Hydraulic system and electrical components of the lift

- Appearance and explosion diagram of the hydraulic station

- Hydraulic system of the lift

A. Hydraulic Schematic Drawing

B. Hydraulic Ellement e

C. Operating principle of the hydraulic system

D. Electrical Schematics Drawing

1. Appearance and explosion diagram of hydraulic station

| SNumber | Name: | Quantity | SNumber | Name: | Quantity |
| --- | --- | --- | --- | --- | --- |
| A1 | Electric motor | 1 | A9 | Seal ket | 1 |
| A2 | Valcore | 1 | A10 | Head head | 1 |
| A3 | Oil tank | 1 | A11 | M5X40 outer hexagon screw | 4 |
| A4 | Down speed control valve | 1 | A12 | Φ5 flat cushion | 4 |
| A5 | Head head | 1 | A13 | Φ5 Ppad | 4 |
| A6 | Oil outlet | 1 | A14 | M6X20 outer hexagon screw | 4 |
| A7 | Oil outlet | 1 | A15 | Φ6 flat cushion | 4 |
| A8 | Hand-wrench valve | 1 | A16 | Φ6 Ppad | 4 |

2.Hydraulic system for the lift

A. Hydraulic Schematic Drawing

B. Hydraulic Element e

| A9 | Oil cylinder |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- |
| A8 | Solenoid valve |  |  | 1 |  |
| A7 | Check valve |  |  | 1 |  |
| A6 | Down-down throttle valve |  |  | 1 |  |
| A5 | Oil pump |  |  | 1 |  |
| A4 | Overflow valve |  |  | 1 |  |
| A3 | Oil tank |  |  | 1 | Volume: 10 L |
| A2 | Oil filter |  |  | 1 |  |
| A1 | Motor: |  |  | 1 |  |
| Number | Name: | Model Number | Specification | Number of parts | Notes |

- The hydraulic system works as follows:

When pressing the start button on the hydraulic station, the motor starts to drive the oil pump and inhale the pressure pump from the fuel tank to the serial number A9 cylinder to move the piston rod. The A4 overflow valve is closed and the pressure is adjusted before delivery. Requirements for rated load for lifting. However, when the system pressure exceeds the limit, the automatic overflow valve discharges the oil. Release the start button, stop oil supply, lift, and start operation. If it drops, click a little first, open the mechanical safety lock, press the drop button and the solenoid valve A8 begins to unload oil.

D. Electrical Schematic (three phase, or single phase)

(* During installation, we must connect according to the schematic diagram to ensure that each connection is reliable and grounded, and the input power supply must be equipped with electric leakage and overcurrent protection switch)

| SB | Point torsion | LXW5-11M/L | 1 |
| --- | --- | --- | --- |
| KM | AC contactor | CJX1-6.3/01-240V | 1 |
| Code name | Name: | Model Number | Quantity |

- Common Troubleshooting

| SNumber | Failure ault | Exclclusion Method |
| --- | --- | --- |
| 1 | Motor is not rotating | ● Check for power ● Check for loose wiring in the motor junction box |
| 2 | Motor rotation without pressure oil | ● single phase electric rotation direction is wrong, changing the two incoming line positions. ● Check the suction pipe in the tank. |
| 3 | Hydraulic drops slowly after lifting (Poor pressure protection) | ● Wash the check and reversing valves for the power unit. |
| 4 | Security lock lock | - Observe the normal position of the lock safety hook. - Observe whether the column position is correct. - Check the reset spring action of the safety hook. |
| 5 | Motor and electrical appliance failure | ●Cut off the power in time and is inspected, repaired and replaced by a professional electrician. |
| 6 | Others | ●Other found to work abnormal, telephone consultation. Willing to serve you. |

- 10. Packing list

|  |  | Boacking |  |
| --- | --- | --- | --- |
| SNumber |  | Product Name | Quantity |
| 51 |  | Power unit | 1 |
| 52 |  | 9 / 16 Tip power unit connector | 1 |
| 53 |  | ￠ 14 Copper sealing gasket | 1 |
|  | 25 | M8X35 outer hexagon bolt | 4 |
|  | 34 | Diameter: 8 flat pad | 8 |
|  | 35 | Diameter: 8 bullet pad | 4 |
|  | 45 | M8 nut | 4 |
| 55 |  | AL-3000-03 large oil and water separator | 1 |
| 56 |  | Right-angle air connector for ￠ 8 | 2 |
|  | 20 | M6 × 20 round-head bolt | 2 |
|  | 32 | Diameter: 6 flat pad | 2 |
|  | 33 | Diameter: 6 bullet pad | 2 |
| 57 |  | Oil and water separator protection bracket | 1 |
|  | 18 | M6 × 15 External hexagon bolt | 2 |
|  | 32 | Diameter: 6 flat pad | 2 |
|  | 33 | Diameter: 6 bullet pad | 2 |
| 58 |  | TSV86522-S pneumatic unlock panel valve | 1 |
| 59 |  | Pass-through air connector for ￠ 8 | 2 |
| 60 |  | Micro-silencer | 2 |
| 61 |  | Plug head | 1 |
| 62 |  | ￠ 8, trachea | 1 |
|  |  | Expansion expansion bolt | 16 |
|  |  | Circular gap flat cushion | 20 |
|  |  | Wire tie | 10 |

- 11. List of vulnerable parts

| SNumber |  | Product Name |  |
| --- | --- | --- | --- |
| 55 |  | AL-3000-03 large oil and water separator |  |
| 58 |  | TSV86522-S pneumatic unlock panel valve |  |
| 76 |  | Platform steel wire rope wheel |  |
| 77 |  | Nylon wear sleeve |  |
| 79 |  | Type of cord core 17.5, L1 |  |
| 80 |  | Type of cord core 17.5, L2 |  |
| 81 |  | Type of cord core 17.5, L3 |  |
| 82 |  | Type of cord core 17.5, L4 |  |
| 85 |  | Beam steel wire rope wheel |  |
| 86 |  | Oil-free bearing |  |
| 92 |  | Beam mechanical insurance limit wheel |  |
| 93 |  | Beam beam slider B-100X30X6 |  |
| 97 |  | Beam beam slider A |  |
| 98 |  | Beam Limit Slider- ￠ 50 ￠ 12 × 11 |  |
| 102 |  | Nylon slip |  |
| 106 |  | Rubber brake rubber pad |  |